Quote Specifications & Weight For:

2027 Titan 48’ 2 Axle Thinwall V Floor 9 Slat Trailer

Customer Application			Garbage Application

Base Model				48-2-TW-VF OT

Body Style				Straight (top panel cut to wedge)

Wall configuration	Front		To suit garbage application upgraded to EHD panels wherever possible

Sides	7-14” + 1-12” to suit garbage application upgraded to EHD panels wherever possible, top panel cut to wedge

Rear Door	Sideswing, Solid, lights in door, air release             w/reinforced Bottom lock finger- See Note #3

Overall Length			48’

Overall Width				102”

Overall Height				13’ 3-78” (159.875”) approx.

King Pin Location			36” WELD IN

Upper Coupler 			5/16” THK- set @ 48” high

Number of Axles	Primary 	Two (2)

Axles					5” Round .580 Wall-Straight Spindle – See Note 1

Axle track				77.5”

Suspension				Titan /Eveley with inner & upper wear pads with 3 leaf

springs

Location of Spreads			See axle layout

Hub & Drum Assembly 		10 Stud Dura-light Hub, Centrifuse Drum

Disc Wheel Size			8.25 x 22.5

Rims					Steel Std Duty (Prepainted White)

Tires			Brand		Goodyear

Size		11R22.5 – 41.4 dia

Tread		Endurance RST

Ply		16

Quantity	8

Axle layout				FRONT–Prim-50-Prim-28-REAR

Oil Seals				To suit axles

Lubricants				Oil-80W-90

Brakes			ABS		2S1M Meritor Wabco

Slack Adj	Meritor Auto

Cam Enclos	All Axles c/w Dust Shields

Lift Kits				None

Lift Control				None

Frame Width				48”

Rear Bumper				Aluminum Offset, Push

Additional Bumper Option # 1	Tow Shackles

Additional Bumper Option # 2	Steel Vertical Blocks Bolted to Bumper protrudes 3 ½” sloped top

Tire Carrier 				None

Landing Gear	SAF-Holland Mark V: 105 Std Skid Foot, Standard hollow axle

Top Rail	Titan Tapered V1

Top Cross Tube	Qty 1- Spring Steel (two piece style) – See Note # 5

Floor Crossmembers	12” Centers

Keith Walking Floor Kit	Keith Running Floor II -3.5” Dia Cylinders, 6” stroke

– See Note # 4

Slat Model & Width 	V9 Slat: 2631 slats  (Alum flat top – 3/8” THK), 2490 Subdeck, 2493 J-Bearing

Slat Wear Strips 	48” long steel tips

Continuous Bearings			None

Side Wall Wear Plates		Boomerang Style

Front Access Door	Std-(35”W X 37.5”H), Curbside Hinge W/ Flip Step, located center

Tarp Style	None

Tarp Header				None

Tarp Basket				None

Tarp Hooks				None

Axle Fender				None

Hyd Fitting		Pressure	1” Quick Coupler – See Note # 2

Return		1” Quick Coupler – See Note # 2

Mud Flaps		Style		Titan Black Rubber

Location	(2) at rear

Mud Flaps – Landing Gear		None

Paint			Body - Color	Natural Aluminum

Run – Gear	Titan Black

Hub/Drum	Same As Running Gear

Rims Or Disc	Steel Prepainted White

Light Package	(Plug Form)		Front: (1) mkr per corner mitre. Sides: (2) mkrs (includes midturn)

Rear: (5) mkrs at top, (3) 4” round taillights per side in Door

Lights Option #1	None

Lights Option #2	None

Access		Catwalk		Universal Roadside Enty 24” Catwalk

J-Ladder – Front	Roadside to Catwalk

J-Ladder – Rear	None

Steps-Front-Out	None

Steps-Front-Ins	None

Steps-Rear		Rung style steps up middle

Additional Specifications:

- a) Invert

b) Axles prepped for PSI Meritor Tire Inflation

2)   a) Add 9’ hoses to front wall

b) Add hinged lid hose hanger to front wall

3)   Tapered door header with 1-PC gusset attached to top side of tapered header. Gusset to suit 10”

bolt on tarp header

4)   Add 2.5” hydraulic pressure gauge to frame (flush mount), located beside the on/off handle with

identifying label

5)    Ship Loose: QTY (1) 4” OD steel crosstube pinned to steel bolt-on brackets

Estimated weight: 19,312 lbs.   +/- 3%

Estimated volume: 127 cu yds

List Price									$ 136,127.72 ea

No FET Included, FOB Chebanse, IL

No State Or Local Tax Included

No License Or Title Fees Included

_____________________________________		_________________

Signature of Acceptance					Date

Ken’s Truck Repair, Inc.