# Quote Specifications & Weight For: 2027 Titan 48’ 2 Axle Thinwall Tipper Trailer

# Spring Ride

Customer Application	Garbage sheet floor

Base Model	48-2-TW-TP (open top)

Body Style	Straight

Panel Configuration	7-14” + 1-6” panels to suit garbage application – See Note # 3 Overall Length	48’

Overall Width	102” Nominal

Overall Height	13’ 4” (160”) approx.

King Pin Location	36” bolt in

Upper Coupler	Aluminum coupler assembly with stainless steel wear plate (3” drop) set @ 48” high

Number of Axles	Primary	(2) Two

Lift	(0) None

Axle	5” Round .580 Wall-Straight Spindle – See Note # 2

Axle Track	77.5”

Suspension	Titan Eveley with inner & upper wear pads with 3 leaf springs

Location of Spreads	See axle layout

Hub & Drum Assembly	10 stud dura-light – hub/centrifuse drum Disc Wheels Size	8.25 x 22.5

Rims	ALUM LVL Outer/Inner

Tires	Brand	Goodyear – See Note # 4 Size	11R22.5 – 41.4 dia

Tread	Endurance RST

Ply	16

Quantity	8

Axle Layout	Front-Primary-50- Primary-29.5-Rear

Oil Seals	To suit axles

Lubricants	Oil 80W-90

Brakes	ABS	2S/1M Meritor Wabco

Slack Adj	DRUM BRAKES: Meritor Auto

Cam Enclos	DRUM BRAKES: All Axles & c/w Dust Shields

Lift Kits	None

Lift Controls	None

Springs	3 Leaf TRA-3384

Frame Width	48”

Rear Bumper	Aluminum, Flush, Narrow

Rubber Bumpers	Tailgate

Aluminum Rubber Channel Bumpers – See Note # 1

Tow Hooks	      Tow Hook welded to rear hanger, Roadside

Tire Carrier	None

Floor	¼” THK alum. sheet & 5/16” THK alum sheet at 6’ of rear Floor Cross members	U channel: 18” centers and 12” centers over subframe Landing Gear	Jost Standard A400 Series: A401-G3-low profile cushion ft

– See Note #1

Top Rail	Titan Tapered,V1

Top Cross Tube Qty	QTY (1)

Top Cross Tube Type	Spring steel (two piece style)

Rear Door	Overslung hinge, door in door, air release & safety chain

Rear Door Seal	Rubber

Front Door	None

Tarp Header	None

Tarp Basket (Hoops)	None

Tarp Hook	Side hooks approx. 54” o/c & (4) hooks at rear

Tarp/Roof	None

Mud Flaps	Style	Titan Black Rubber

Location	(2) ahead & (2) behind primary group

Paint	Body	Natural Aluminum Run – Gear	Titan Black Hub/Drum	Same as running gear Rims or Disc Aluminum

Std Light Package – plug form	Front (2) mkrs. Sides: (3) mkrs (included midturn) Rear: No mkrs

at top w/center cluster of (3) mkrs in lightbar, (2) 4” round red taillights per side

Option #1	None

Option #2	None Access	Catwalk – front	None

Ladder – front	Middle of frontwall full height

Ladder – rear	QTY (1) middle full height with (1) small catwalk on each side Steps – front – out	None

Steps – front – ins	None Steps – rear	None

Additional Specifications:

- Approx. 34’ 11-¾” from center of landing gear to rear of rubber bumpers

- Invert Axles

- Boomerang wear plates

- Add PSI Meritor Tire Inflation System to all axles

Estimated volume: 125 cu yds Estimated weight: 12,743 lbs +/- 3%

List Price										$ 91,588.22 ea

No FET Included, FOB Chebanse, IL

No State Or Local Tax Included

No License Or Title Fees Included

Signature of Acceptance	Date

Ken’s Truck Repair, Inc.